_______________________________

|                               |

|   DESIGN CENTER - SYSTEM 3    |

EVALUATION VERSION

|         5.3 RELEASE           |

|_______________________________|

This file describes any enhancements or modifications made to the Evaluation

version of the 5.3 release of the Design Center - System 3 and covers new

information for the IBM-PC Microsoft Windows, and Sun OpenWindows platforms.

The Design Center - System 3 Evaluation version includes Schematics, PSpice,

Probe, the Stimulus Editor, and Parts programs.  Information regarding new

material for these programs and any material which did not make it into the

manuals is provided in this file.

With the Design Center - System 3 Evaluation version, the Schematic Editor

is limited to one page schematics, A-size pages only, and a maximum placement

of 20 parts on the page.  The maximum circuit size is 64 nodes and 10 active

components (or approximately 2 opamps).  Only a subset of our Model and Symbol

Library is included, and the device characterization utility is limited to

modeling diodes.

The Design Center - System 3 manual set includes the following:

Schematic Capture User's Guide

Circuit Analysis User's Guide

Circuit Analysis Reference Manual

Circuit Analysis System Setup Manual

Design Center Application Notes Manual

These Manuals will be referenced throughout this file.  If you would like to

purchase the manuals or the complete production version of the Design

Center, please call MicroSim at (800) 245-3022.

****************************************************************************

MicroSim Corporation now offers the Technical Support Bulletin Board

System (BBS).  On the BBS you will find information regarding the

operation of our Design Center systems.  You can request new literature

packages, and use the Message utility to let us know your comments and

suggestions.

The BBS supports several different terminal types, but is basically set

up for the 80 character wide, 24 line, ANSI and VT100/VT102 terminal

emulation.  You may log on to the system as often as you like, but

logon time is limited to 60 minutes per day.  The system will automatically

log you off when left idle for 5 minutes.  We encourage you to log on

and have a look around!

To access our BBS, dial:

714-454-7611

{ 9600-1200, N-8-1 }

*********************************************************************

Table of Contents

*********************************************************************

** SCHEMATICS **

1.0  Editing Model Definitions in Schematics

2.0  Specifying Digital Power Supplies

2.1  Using the Stimulus Generator and File Stimulus Device in

Schematics

2.2  HI and LO digital nets

3.0  Interfacing to External Layout Packages

3.1  Protel for Windows

4.0  Schematic Editor New Menus and Menu Items

4.1  Edit Menu

4.2  Navigate Menu

4.3  Configure Menu

4.4  Analysis Menu

4.5  Tools Menu (New Menu)

5.0  Symbol Editor New Menus and Menu Items

5.1  Edit Menu

5.1.1  Changes to the "Pinlist" and "Change Pin" dialogs

5.2  Packaging Menu (New Menu)

6.0  Layout Mapping Files

7.0  Library Expansion and Compression Utilities

8.0  Parts With Unmodeled Pins

8.1  How the PSpice Netlister Handles Unmodeled Pins

8.2  Floatable Pins

8.3  Implementation of Unmodeled and Floatable Pins

9.0  Attribute Values and Hierarchy in Schematics

10.0  Known Problems and Workarounds

10.1  Hidden Pin Display

11.0  Example Schematics

11.1  example.sch

11.2  osc.sch

** PSPICE **

1.0  PSpice Commands

1.1  .MODEL

1.2  GaAsFET, Capacitor, Diode, JFET, Inductor, MOSFET, Bipolar Transistor,

and Resistor (new parameters)

1.3  Voltage-Controlled Voltage Source (additional general form)

1.4  .OPTIONS

1.5  .FUNC

2.0  PSpice Model Library Changes

3.0  Miscellaneous Changes to PSpice Simulation

3.1  Flip-Flops and Latches

3.2  Timing Violations

3.3  Pin-to-Pin Delay

3.4  Constraint Check

3.5  Limits on Mutual Inductance

3.6  Using the Charge Storage Feature of PSpice Digital Simulation

4.0  CMOS Power Supplies

4.1  Specifying Your Own Power Supply

5.0  Digital Device I/O Model Parameter Updates

6.0  Remodeled Digital Devices and Timing Capabilities

7.0  Simulation Condition Messages

8.0  Charge Conservation

** PROBE **

1.0  Probe New Menu Structure

*********************************************************************

SCHEMATICS

*********************************************************************

Note on transferring files between PC and Sun computers

Both symbol library files (.slb) and schematic files (.sch) include

a byte count and should therefore NOT be modified using a text

editor.  When transferring files from one system to the other, you

must transfer the file in binary mode (not ASCII).

1.0  Editing Model Definitions in Schematics

Schematics now supports Monte Carlo, Worst-Case, and model editing by

providing a mechanism for manipulation of .MODEL and .SUBCKT definitions

of parts.  Schematics allows you to edit model definitions, create new

models, associate model definitions with symbols, or derive new model

definitions from existing ones.  For information on the implementation

of model editing in the Schematic Editor and Symbol Editor, refer to

Chapter 5, Section 5.4 of the Schematic Capture User's Guide.  Section

5.4.7 includes an example of model editing using Monte Carlo analysis.

2.0  Specifying Digital Power Supplies

Section 5.9 (starting on page 108) in the Schematic Capture User's

Guide describes how to change the power supply on devices from

the digital symbol libraries.

2.1  Using the Stimulus Generator and File Stimulus Device in Schematics

There are two ways to specify a digital input signal in

Schematics.  One is to use the file stimulus device, FSTIM.  The

other is to use a digital stimulus generator.

The file stimulus device is used to access one or more signals

inside a stimulus file.  The Schematics symbol for the file

stimulus is called FileStim and is in the SOURCE.SLB symbol

library.  Double-click on the symbol to enter values for the

FileName and SigName attributes.  FileName is the name of your

stimulus file, and SigName is the name of the signal.  After

connecting the FileStim device to the input node of the circuit,

you can use the Notepad text editor in Windows to create the

stimulus file.  The following is an example of a stimulus file

and shows the proper format:

CLOCK

0         0

20ns      1

40ns      0

60ns      1

80ns      0

Another way to specify a digital input is to use a digital

stimulus.  The digital stimulus generator allows you to create a

variety of digital signals using simple commands.  To use the

digital stimulus requires a few more steps than to use the FSTIM

method.  Place a GLOBAL symbol from the PORT.SLB symbol library

at the digital input node of the circuit.  Label the node by

double-clicking on the GLOBAL symbol.  The label should be the

same name as the node name in the digital stimulus file.  You can

use the Notepad text editor in Windows to create the digital

stimulus file.  The digital stimulus file has the following

format:

U<name> STIM(<width>,<format array>)

+ <digital power node> <digital ground node>

+ <node>*

+ <I/O Model name>

+ [IO_LEVEL= <interface subckt select value>]

+ [TIMESTEP=<stepsize>]

+ <command>

Here is an example of a digital stimulus:

UCLK STIM(1,1) $G_DPWR $G_DGND

+ CLOCK

+ IO_STM

+ 0S 0

+ LABEL=CYCLE

+ +20NS 1

+ +20NS 0

+ +20NS GOTO CYCLE -1 TIMES

After saving the digital stimulus file, use the INCLUDE symbol

from SPECIAL.SLB to include the filename of the digital stimulus.

Both the digital stimulus generator and the file stimulus device

are discussed in more detail in chapter 7.4 of the Circuit

Analysis User's Guide.

2.2  HI and LO digital nets

The HI and LO port symbols enable you to speed up digital simulation

when you want a digital pin tied high or low.  These symbols may be

used across all digital families and will provide a logic 1 or 0.

These symbols are not useable for PCB design, however.  You will need

to replace them with a resistor, if appropriate, and a connection to

power or ground.

3.0  Interfacing to External Layout Packages

Schematics currently supports netlisting for the PADS and Protel

PCB layout editor formats.  The layout netlists produced by Schematics

can be used as input to any external PCB layout package that accepts

these formats.  Schematics also supports file-based backward Engineering

Change Orders (ECOs) in the PADS layout editor format. That is, ECO files

produced by the layout packages can be read by Schematics to back

annotate the schematic.  Schematics can also be configured to directly

invoke an external layout editor from within the schematic environment.

See Chapter 8 of the Schematic Capture User's Guide for more

information on Schematics' interface to external layout editors and

how to package schematics.

3.1  Protel for Windows

In addition to supporting the PADS board layout system, Schematics also

supports Protel for Windows.  To select Protel as the layout editor,

invoke Tools/Configure Layout Editor, click in the 'Layout Netlist'

combo box, and select PROTEL.  You may also configure the command

line used to invoke the editor in the same dialog.

Schematics will produce a netlist in native Protel format.  Three

mapping files ("protel.xmp," "protel.xpk," and "protel.xnt") translate

Schematics output to Protel-specific names (for example, Protel

has a package type 'TO-3' which is 'TO3' to Schematics).

Protel does not have an ECO mechanism, so Schematics does not

provide an ECO function when used with Protel.

Many of the package types used by analog parts in the PSpice

libraries are not present in the Protel pattern library (for

example, there is no 'TO-61' pattern).

4.0  Schematic Editor New Menus and Menu Items

4.1  Edit Menu (new commands)

Model allows you to edit the PSpice .MODEL or .SUBCKT definition for a part.

For complete instruction on using the Edit/Model command, see page 166

of the Schematic Capture User's Guide.

Change Model Reference brings up a dialog that allows you to enter the name

of another model for this part instance.  See page 167 of the Schematic

Capture User's Guide for more information on this command.

Edit Instance Model allows you to edit a copy of the model or subcircuit

definition.  See page 167 of the Schematic Capture User's Guide for

more information on this command.

4.2  Navigate Menu (new commands)

Edit Schematic Instance allows you to view the instance-specific

attributes associated with the instance of the block or hierarchical

symbol into which you are pushed.  See page 172 of the Schematic

Capture User's Guide for more information on this command.

Edit Schematic Definition allows you to edit the underlying schematic

into which you are pushed.  Changes made will affect all instances of

hierarchical blocks and symbols that reference this schematic.  This

command is available only if you have pushed into a subschematic.

4.3  Configure Menu

The Library Settings dialog box has been changed.  If the library you are

adding is a symbol or package library, click in the box respective to

the file type, and the default extension will appear.  See page 176 of

the Schematic Capture User's Guide for an illustration of the new dialog.

4.4  Analysis Menu (new commands and changes)

Annotate assigns reference designators to parts, or deletes previous

annotations.  See pages 177-179 of the Schematic Capture User's Guide

for information on using the commands in this dialog.

Setup brings up a dialog which allows you to enable, disable, and select PSpice

analyses.  To enable an analysis type, click in the Enabled box to the

left of the button of the desired analysis.  This will put a small "x"

in the box indicating that the analysis is turned on.  When the box is empty,

the analysis is disabled.  Pressing OK will exit this dialog.

Library and Include Files allows you to specify files to be included or

referenced in the PSpice netlist.  See page 193 of the Schematic Capture

User's Guide for information on using this command.

4.5  Tools Menu (new menu)

The Tools Menu allows you to interface to external layout editors.  You can

create netlists that can be used as input to your layout editor, and apply

backward ECO's generated by the layout editor to the schematic.  For

details on the new Tools Menu and the commands, see pages 194-195 of the

Schematic Capture User's Guide.

Tools/Configure Layout Editor

Command Line

Command Line specifies the command to be invoked when the Tools/Layout

Editor menu item is chosen.  This command may contain the following

special sequences on the command line:

Code	Description

%c	current working directory

%e	schematic name without extension

%f	entire name

%n	schematic name without extension or path

For example, if the schematic c:\msim53\example.sch is the currently

loaded schematic, then the following text is substituted in the command

line in place of the special codes:

%c	c:\msim53

%e	c:\msim53\example

%f	c:\msim53\example.sch

%n	example

5.0  Symbol Editor New Menus and Menu Items

5.1  Edit Menu (new command)

Model allows you to edit the PSpice .MODEL or .SUBCKT definition for a part.

From within the Symbol Editor, the "global" model or subcircuit representation

is edited.  By default theses changes are saved in a file called "user.lib."

Changes made to the model or subcircuit in the Symbol Editor will affect all

the parts that use this model or subcircuit.  See pages 211-213 in the

Schematic Capture User's Guide for information on using the Edit/Model

command in the Symbol Editor.

5.1.1  Changes to the "Pin List" and "Change Pin" Dialogs

The Pin List and Change Pin dialogs (Part/Pinlist and Edit/Change)

have a new checkbox and a new list box in the Pin Attributes

section.

The checkbox is labeled "Modeled Pin" and is checked by

default.  If the pin is modeled, this box should be checked.  If

the pin is NOT part of the simulation model (such as balance and

offset pins on some opamps), then the box should be unchecked.

When the box is unchecked, the pin is marked as not simulated and

a "FLOAT=u" attribute is applied to the pin.  This allows the

PSpice netlister to pay special attention to the pin in case

there are fewer than two connections at its node.  In that case, a

large resistor to ground is added for simulation purposes.

If the checkbox is unmarked, the "FLOAT=" list box is not

enabled, as noted by the "If Unconnected:" label.  However, if it

is marked, the list box is enabled.  This allows you to pick a

default action that the PSpice netlister will take in the event that

you have not connected this pin to any others.  The choices are

"RtoGND", "UniqueNet" and "Error".  Selecting "RtoGND" will tell

the netlister to add a large resistor to ground for the pin.

Selecting "UniqueNet" will cause a unique node to be generated for

the pin which can be used for seeing its trace in Probe by adding a

marker.  Selecting "Error" will cause the netlister to fail with

an "Unconnected Pin" error message if the pin is not connected to

any others.  (See also Section 9.0 "Parts With Unmodeled Pins" in

the SCHEMATICS section of this file.)

5.2  Packaging Menu (new menu)

The Packaging Menu allows you to edit package definitions in the package

library corresponding to the current symbol library.  Within this menu you

can edit package types, gate types, pin assignments, shared pins, and

attributes associated with the package definitions.  For information

on the new commands in the Packaging Menu, see pages 220-223 in the

Schematic Capture User's Guide.

6.0  Layout Mapping Files

Mapping files contain rules for translating from Schematics to layout system

identifiers. Map files exist for each of the layout formats supported.

See Appendix B in the Schematic Capture User's Guide for information

on mapping files.

The operation of the ` (backquote) character in layout netlist

mapping entries has been changed from what is currently written in the

Schematic Capture User's Guide.  It now operates as a modifier on

'special characters' such as '@'.  It still means "the value of

the attribute when translated by a rule in the .xmp file".  The

table below shows the special character-attribute syntax with and

without the backquote modifer:

@<id>     replaced by value of <id>.  Error if no such attribute or no value.

@`<id>    replaced by mapped value of <id>.  Same error conditions.

&<id>     replaced by value of <id> if <id> defined.

&`<id>    replaced by mapped value of <id> if <id> defined.

7.0  Library Expansion and Compression Utilities

The library expansion utility lx (lx.exe on the PC, lx on the Sun) and

the library compression utility lc (lc.exe on the PC, lc on the Sun) are

provided with the Design Center - System 3 package.  These utilities work

with both symbol and package libraries.

They can be used for:

-  salvaging a corrupted library

-  maintaining library files

-  reorganizing a library

-  creating a batch library

For information on how to use the library expansion and compression

utilities, refer to Appendix C of the Schematic Capture User's Guide.

8.0  Parts with Unmodeled Pins

The Schematics symbol libraries have been updated to include pins

that are not modeled by the parts in the corresponding PSpice

model libraries.  These include pins for offset nulling, etc.

Such pins are termed "unmodeled" pins.

Schematics will display an unmodeled pin with a break in the

graphics for the pin.  That is, a regular pin is displayed as

'---' (a solid pin), whereas an unmodeled pin is displayed as

'- -' (a broken pin).  This distinction applies only to the

display of these pins in Schematics; i.e., these pins will appear

as [regular] solid pins when you print/plot to your hardcopy

device.

Parts which have been changed for the 5.3 release include

operational amplifiers in the analog libraries, and one-shots in

the digital libraries.  Where possible, the location of existing

pins has not been changed such that an existing schematic can be

read in without changes.  Please see Section 8.0 "Library Changes"

of this readme for a list of affected parts.

8.1  How the PSpice Netlister Handles Unmodeled Pins

If an unmodeled pin is left floating (i.e., no connections are

made to it), no PSpice netlist entries will be generated

referring to that pin.

If connections are made to an unmodeled pin, Schematics will

issue an ERC warning but will proceed to generate a netlist.  A

large value resistor will be added to the netlist between the net

connecting to the pin and analog ground.  The netlist line for

this resistor is of the form

R__UC<number>  <node>  0   <value>

The <value> field defaults to {1/GMIN}.  This can be modified

by setting LARGERESISTOR in the [SCHEMATICS] section of msim.ini

to the desired value.

Note that GMIN is one of PSpice's built-in global parameters.

Its value can be modified by using the Analysis/Setup/Options

dialog in Schematics.  The default value will result in a

resistor of value 1E12 ohms.

8.2  Floatable Pins

The Schematics Symbol library has been modified to allow pins

on digital symbols to float.  This is considerably more

convenient than having to connect unused pins to the "NC"

pseudocomponent.  It also allows digital levels at these pins to

be viewed by placing a Probe marker on the pin.

When a "floatable" digital pin is left unconnected, the netlister

will connect that pin to a unique net.  No other pins will be

connected to this net.

It is also possible to specify that a pin is floatable and

analog.  For floatable analog pins, a large value resistor will

be added to the netlist between the pin and analog ground, when

the pin is left floating.  [See above section for details].

8.3  Implementation Of Unmodeled and Floatable Pins

Please see the Section 5.1.1 of this readme file for

details.

Note that if a pin is defined to be unmodeled, there should be no

reference to the pin in the PSpice netlist TEMPLATE.

9.0  Attribute Values and Hierarchy in Schematics

A number of changes have been made to the way that attributes

are evaluated in Schematics.

First, if an attribute is not found at the current level of

hierarchy, then the parent level is searched for a definition,

continuing up the hierarchy until either a definition is found or

the top of the hierarchy is reached.  (Previous versions of

Schematics searched only the immediate parent's environment.)

Secondly, when an attribute is found, it is evaluated at the level

where it was found.  If the attribute value contains further

attributes, these must exist at the current level or higher in the

hierarchy.  For example, suppose that we have an instance of a

hierarchical symbol 'A' which has defines two attributes: X=@Y and

Y=10.  Suppose also that A contains an instance of a symbol 'B':

B contains an expression referring to the attribute X and defines

the value of attribute Y to be 20.

-----------------------

|	   A		| X=@Y

|	   		| Y=10

|       ---------       |

|      |    B    |      |

|      |  {@X}   |      |

|      |  Y=20   |      |

|       ---------       |

-----------------------

Evaluation of the expression {@X} takes place as follows.  The

current level is searched for X.  There is no X attribute at this

level, so the parent environment (part A) is searched.  An attribute

named X is found at this level.  This attribute is evaluated IN THE

ENVIRONMENT SUPPLIED BY A.  The first stage of this evaluation

delivers the result "@Y".  This is then processed to yield the

result "10".  The final result is to make the result of the

expression be "{10}".  Note that the definition for Y in the

environment supplied by part B must not be used when evaluating

X in A's environment.  (Previous versions of Schematics did this.)

In practice this means that the limitation where attributes were

looked for in only one level of hierarchy above the current level

has been removed.  This makes it possible to parameterize hierarchical

blocks and symbols without concern for how deeply their contents are

nested.

10.0  Known Problems and Workarounds

10.1  Hidden Pin Display

The display of hidden pins will incorrectly be turned on when you use

the Edit/Push command from within the Symbol Editor, Navigate/Pop when

finished, and then return from the Symbol Editor to the Schematic

Editor and exit.

The workaround is to use Configure/Set Display Level to reset the

display of hidden pins to OFF before you exit.

11.0  Example Schematics

11.1  example.sch

example.sch is a differential pair, and shows many of the capabilities

of PSpice.  Select Analysis/Setup to examine some of the different

ways PSpice can analyse this circuit.

The AC Sweep button will show you the AC and NOISE analysis setup.

The real and imaginary response of the circuit is calculated as the

inputs are swept from 100 kilohertz to 10 gigahertz by decades with

10 points per decade.  The only AC input this circuit has is V1.

This is a linear analysis.  Enabling NOISE analysis (with AC analysis

enabled) will cause PSpice to do noise calculations during the AC

analysis.  Each device's noise contribution is calculated and

propogated to node OUT2.  All the contributions are rms-summed at

node OUT2.  Besides the total output noise printout done for every

frequency, a detailed table of each device's contribution is done

every 30'th frequency.

The DC Sweep button shows you the setup for DC analysis.  The voltage

source V1 is swept from -0.125 volts to 0.125 volts in steps of 0.005

volts.  The non-linear device equations are used.

Pushing the Temperature button (with enabled checked) tells you that

PSpice is being told to simulate the circuit at 35 degrees Celcius.

Push the Transfer Function button to see the setup for doing a

small-signal transfer function calculation assuming V1 is the input

and V(OUT2), the voltage at node OUT2, is the output.

The Transient button (enabled) causes PSpice to do a transient analysis.

PSpice first re-calculates the circuit's bias point, then calculates

the circuit's time response from 0 nanoseconds to 5 microseconds using

the full, non-linear device equations, including non-linear capacitances.

PSpice uses a variable time step for the calculations, but this command

causes the results to be interpolated onto a 20 nanosecond print

interval.  Transient analysis is the most frequently used analysis in

PSpice.

The Fourier Analysis (enabled within the Transient dialog) tells PSpice

to do a harmonic decomposition on the waveform V(OUT2) calculated

during transient analysis.  It calculates the magnitude and phase of

the fundamental (1 megahertz) and the first eight harmonics.  The

graphics post-processor, Probe, goes further and contains a full FFT,

allowing complete spectra to be displayed.

11.2  osc.sch

osc.sch is a mixed-mode ring-oscillator circuit.  It consists of a pair

of schmitt-trigger inverters, a 2K resistor, a 400pF capacitor.  A

digital stimulus is used to start the oscillation and clear the toggle

flip-flop at the output.  The 74LS05 open-collector inverter floats

after the initial 0 pulse occurs.

*********************************************************************

PSPICE

*********************************************************************

1.0  PSpice Commands

1.1  .MODEL

PSpice now allows for the customization of model temperature

for passive and semiconductor devices.  There are two levels of

temperature attributes which can be customized on a model by model

basis; the temperature at which the model parameters are assumed to

be measured and current device temperatures.  See page 60, and pages

62-64 of the Circuit Analysis Reference Manual for information on

temperature customization.

1.2  GaAsFET, Capacitor, Diode, JFET, Inductor, MOSFET, Bipolar Transistor,

and Resistor (new parameters)

Model Param.    Description                           Units      Default

------------    -----------------------------------   --------   -------

T_MEASURED      Measured temperature                  degrees       C

T_ABS           Absolute temperature                  degrees       C

T_REL_GLOBAL    Relative to current temperature       degrees       C

T_REL_LOCAL     Relative to AKO model temperature     degrees       C

For information on these parameters, see the discussion above about the

.MODEL statement.

1.3  Voltage-Controlled Voltage Source (additional general form)

E<name> <(+) node> <(-) node> CHEBYSHEV { <expression> } =

+ <[LP] [HP] [BP] [BR]>,<cutoff frequencies>*,<attenuation>*

Chebyshev filters have two attenuation values, given in dB, which

specify the pass band ripple and the stop band attenuation.  They may

be given in either order, but must appear after all of the cutoff

frequencies have been given.  Low pass (LP) and high pass (HP) have two

cutoff frequencies, specifying the pass band and stop band edges, while

band pass (BP) and band reject (BR) filters have four.  Again, these may

be given in any order.

1.4  .OPTIONS Command

.OPTIONS (new options)

Option          Meaning                               Units      Default

------          -----------------------------------   -----      -------

NOOUTMSG        Suppress simulation error messages

in output file.

NOPRBMSG        Suppress simulation error messages

in Probe data file

DIGERRDEFAULT   Default error limit for digital

constraint devices

DIGERRLIMIT     Maximum digital error message limit             infinite

1.5  .FUNC

Please note that the .FUNC statement expects its arguments to be of

numerical values (such as V(a) or V(5)) instead of numerical forms

of the names (such as node names).

The example on page 43 of the Design Center Application Notes Manual

does not work.  The following information should be changed:

Change	.FUNC vsq(node) v(node)*v(node)

to	.FUNC vsq(nodev) nodev*nodev

Change	g1 vd 0 value {k*((-v(vo)*v(vf))+(ve(vo,vd,va)*v(vd)))}

to	g1 vd 0 value {k*((-v(vo)*v(vf))+(ve(v(vo),v(vd),v(va))*v(vd)))}

2.0  PSpice Model Library Changes

See Section 8.1 of the SCHEMATICS Section of this file.

3.0  Miscellaneous Changes to PSpice Simulation

3.1  Flip-Flops and Latches

X-Level Handling

The truth-table for each type of flip-flop and latch is given on page

206 of the Circuit Analysis Reference Manual.

3.2  Timing Violations

The Circuit Analysis Reference Manual now describes the conditions

such as timing violations, which cause the flip-flop and latch

primitives to change their outputs to X.  In addition, the truth

tables for each type have been added.  Refer to page 206 for

more information.

3.3  Pin-to-Pin Delay

The PINDLY primitive has been enhanced to model tri-state output

behavior.  Refer to pages 236-244 of the Circuit Analysis Reference

Manual for new the new syntax.

3.4  Constraint Check

The CONSTRAINT primitive now allows you to specify RELEASETIME parameters.

In addition, it supports distinct LO and HI level SETUP and HOLD time

specifications.  See Section 5.1.1.11.3 "Constraint Checker" on page

245 of the Circuit Analysis Reference Manual for the new syntax.

3.5  Limits on Mutual Inductance

The limits on mutual inductance (often called the coupling coefficient)

for K devices has been changed.  The old limits were:

0 < x < 1

The new limits are:

-1 <= x <= 1

Note that besides the extension from 0 to -1, the range now includes

the extremes.

3.6  Using the charge storage feature of PSpice digital simulation

The ability to model charge storage on digital nets is new for PSpice

version 5.3, and is not included in the 5.3 documentation.  This feature

is mainly useful for engineers who are designing dynamic MOS integrated

circuits.  In such circuits it is common for the designer to temporarily

store a one or zero on a net by driving the net to the appropriate voltage

and then turning off the drive.  The charge which is trapped on the net

causes the net's voltage to remain unchanged for some time after the net

is no longer driven.  The technique is not normally used on PCB nets because

sub-nanoamp input and output leakage currents would be required, as well as

low coupling from adjacent signals.

PSpice models the stored charge nets using a simplified "switch-level"

simulation technique.  A normalized (with respect to power supply) charge

or discharge current is calculated for each output or transfer gate attached

to the net.  This current, divided by the nets total capacitance, is

integrated and recalculated at intervals which are appropriate for the

particular net.  The net's digital level is determined by the normalized

voltage on the net.  Only the digital level (1, 0, R, F, X) on the net is

used by device inputs attached to the net.

This technique allows accurate simulation of networks of transfer gates and

capacitive loads.  The sharing of charge among several nets which are

connected by transfer gates is handled properly because the simulation

method calculates the charge transferred between the nets, and maintains

a floating-point value for the charge on the net (not just a one or zero).

Because of the increased computation, it takes PSpice longer to simulate

charge storage nets than normal digital nets.  Charge storage nets are

simulated much faster than analog nets, however.

In order for PSpice to decide which nets (if any) should be simulated as

charge storage nets we have added three new model parameters to the digital

I/O model (model type UIO).  They are:

Parameter	Description			Default Value

______________________________________________________________

INR		Input leakage resistance	30Kohm

DRVZ		Output Z-state leakage		250Kohm

resistance

TSTOREMN	Minimum storage time for net 	1.0mSec

to be simulated as a charge

storage net.

PSpice will simulate charge storage only for a net which has some devices

attached to it which can be high Z, and which has a storage time greater

than or equal to the smallest TSTOREMN of all inputs attached to the net.

The storage time is calculated as the total capacitance (sum of all INLD

and OUTLD values for attached inputs and outputs) multiplied by the total

leakage resistance for the net (the parallel combination of all INR and

DRVZ values for attached inputs and outputs).

The default values provided by the UIO model will not allow most user

circuits, even those which use non-MicroSim libraries of digital devices,

to use the charge storage simulation techniques.  This is appropriate,

since these libraries are usually for PCB-based designs.

4.0  CMOS Power Supplies

The CD4000 CMOS models now use different default power supply nodes:

$G_CD4000_VDD and $G_CD4000_VSS.  The voltage on these nodes can now

be set directly by adding a .PARAM statement to your circuit.

4.1  Specifying Your Own Power Supply

Designs using CD4000 devices often require power supply voltages other than,

or in addition to, the default 5.0 volts supplied by CD4000_PWR.  The Digital

Library offers two methods for changing the power supply voltage on CD4000

series CMOS parts.  For details on both methods, refer to Section

7.3.2 of the Circuit Analysis User's Guide (page 123).

5.0 Digital Device I/O Model Parameter Updates

Non-zero values of INLD and OUTLD will now cause nets with tri-state or

bidirectional transfer gates to be simulated as charge storage nets.

6.0 Remodeled Digital Devices and Timing Capabilities

The Digital Library for version 5.3 contains many remodeled devices.  Most of

the complex sequential device models, such as counters and shift registers,

now use the Logic Expression (LOGICEXP), Pin-to-Pin Delay (PINDLY), and

Constraint Checking (CONSTRAINT) primitives, which were introduced in version

5.2.  These new models offer improved readability, improved timing accuracy,

and complete checking of setup/hold times and pulse widths.  Any timing

violations on these models are reported in detail as warning messages to

the PSpice output file (".out"), and to the Probe data file (".dat") for

use by Probe (currently, only PC/Windows Probe will display these messages).

These warning messages are discussed further in the next section of this

"readme" file, and also in Section 1.4 ".OPTIONS Command."

A side benefit of the new Digital Library models is that they will NOT

generate X states (unknowns) on the output when their timing constraints are

violated.  This new behavior makes it possible for your simulations to run to

completion even when there are violations, without having everything

"lock up" with X states.  This can be very helpful early in the design

process when circuit timing is a lesser concern than just getting the basic

functionality to check out.

It is important to note that, when PSpice reports timing constraint

violations, the actual behavior of the circuit (despite the absence of X

states) should be considered "possibly incorrect."  The only sure way to

obtain the full degree of accuracy embodied in the model is to resolve the

timing issues by making appropriate circuit and/or stimulus changes,

effectively removing all violation conditions.

Also note that any circuits or library subcircuits that use flip-flop or

latch primitives (DFF, JKFF, SRFF, or DLTCH) will continue to generate

X states when their timing specifications are violated.  They will also

produce warning messages.  This is inconsistent with the behavior

of the Constraint Checking primitive, and will be addressed in a future

release.

7.0  Simulation Condition Messages

PSpice will produce warning messages in various situations, such as

those that originate from the digital CONSTRAINT devices

monitoring timing relationships of digital nodes.  These messages are

directed to the PSpice output file and/or to the Probe data file for use by

Probe (currently, only PC/Windows Probe will display these messages).

Options are available for controlling where, and how many of these messages

are generated, as summarized in section 9.7 (starting on page 193)

of the Circuit Analysis User's Guide.   In version 5.3, messages describing

"Net Conflicts" and "Suppressed Glitches" will NOT be produced by PSpice.

All other Hazards and Timing Violations will generate messages, by default.

8.0  Charge Conservation

For MOSFETs the capacitance model has been changed to conserve

charge.  This change affects the level 1, 2, and 3 models.  The

level 4 (BSIM) model has its own capacitance model which already

conserves charge and remains unchanged.

The Meyer equations for levels 1, 2, and 3 were replaced by the

Yang-Chatterjee equations which are described in:

"An investigation of the Charge Conservation Problem for

MOSFET Circuit Simulation" by Ping Yang, Berton Epler and

Pallab Chatterjee, IEEE Journal of Solid-State Circuits, vol.

SC-18 no. 1, February 1983.

Care was taken to see that (all) the charges are continuous across

the boundaries.  The resulting behavior matches closely the Meyer

model.  The main difference is that charge is now conserved.

*********************************************************************

PROBE

*********************************************************************

1.0  Probe - New menu structure

Information about all the changes in the Windows Probe menus

can be found in Chapter 6 of the Circuit Analysis Reference Manual.

See the page numbers referenced below to find out information about

each main menu and the items within them.

File Menu handles all of the file manipulation: opening plot windows and

files, closing files, printing, and logging commands. (Page 328)

Edit Menu provides commands to delete or modify objects. (Page 332)

Trace Menu allows adding traces, creating macros, and evaluating goal

functions on a single section of data.  (Page 332)

Plot Menu allows you to switch to a different analysis type, add and

delete plots, and modify the X and Y axes.  (Page 340)

Zoom Menu provides a quick and simple, visual way to change the X and Y

ranges of the current plot. (Page 343)

Tools Menu allows you to add additional information to the plot window,

to save and restore the display, and to configure Probe.  (Page 344)

Window Menu allows you to perform operations on plot windows.  The

Window Menu is currently not available for OpenWindows Probe.

(Page 347)

Help Menu allows you to get user identification and copyright

information. (Page 348)